1. System Architecture

Overview

The proposed Smart Waste Bin Network is an IoT-enabled waste management system designed to monitor garbage fill levels in real time, notify authorities before overflow, and optimize waste collection routes using data analysis and prediction techniques. The system follows an end-to-end IoT architecture consisting of sensing, processing, communication, cloud computing, and visualization layers.

Hardware Components

Sensor:
An ultrasonic sensor is mounted at the top of each waste bin to measure the distance between the sensor and the garbage surface. This distance is converted into a percentage fill level based on the total depth of the bin.

Microcontroller:
An ESP32 microcontroller is used to interface with the ultrasonic sensor. It performs local processing, data formatting, and wireless transmission. ESP32 is selected due to its low power consumption, integrated communication support, and suitability for IoT applications.

Communication Method:

For city-wide deployment, LoRa (Long Range communication) is proposed due to its:

- Long communication range

- Low power consumption

- Suitability for battery-operated devices

Alternatively, Wi-Fi or NB-IoT can be used for smaller or infrastructure-rich areas.

Cloud & Computing Strategy:

All bin data is transmitted to a cloud platform such as AWS IoT, Firebase, or ThingsBoard. The cloud performs:

- Data storage

- Threshold-based alert generation

- Historical data analysis

- Prediction of future bin fill levels

Edge processing is limited to basic filtering, while analytics and prediction are handled centrally.

Dashboard & Visualization

A web-based dashboard is provided for municipal authorities. It displays:

- Real-time bin fill levels

- Alert notifications

- Zone-wise bin status`

- Map-based visualization for route planning

- Data Flow Design

- The data flow design ensures continuous monitoring, intelligent prioritization, optimized route planning, and real-time status updates for smart waste management.

- The system continuously monitors the fill level of waste bins using ultrasonic sensors installed in each bin.

The data flow design ensures continuous monitoring, intelligent prioritization, optimized route planning, and real-time status updates for smart waste management.

The system continuously monitors the fill level of waste bins using ultrasonic sensors installed in each bin.

- Sensor data is processed locally by the ESP32 microcontroller and transmitted periodically to the cloud platform.

- The received data is stored in the cloud as both real-time and historical information for further analysis.

- Based on predefined thresholds, the system classifies bin status using color indicators:

Green for low fill level

Yellow for moderate fill level

Red for high fill level requiring immediate attention

Bins with a minimum fill level (≥25%) are selected for waste collection planning.

The system generates optimized collection routes by grouping nearby bins that require service.The optimized route is assigned to the garbage collection truck to minimize travel distance and fuel consumption.After waste collection, bin fill levels are reset to zero and their status is updated in the system.The updated bin status is reflected on the dashboard for monitoring by municipal authorities.The entire data flow operates continuously to ensure efficient, scalable, and timely waste management.

In the Smart Waste Bin Network, reliable and efficient communication is required between smart bins, cloud servers, and user dashboards. The following protocols are identified and used based on system requirements such as low power consumption, scalability, and real-time updates.

1. MQTT (Message Queuing Telemetry Transport)

Where it is used:

- Between Smart Bins (ESP32) and the Cloud Platform

- For sending bin fill-level data and receiving control messages

Why MQTT is suitable:

- Lightweight protocol, ideal for low-power IoT devices

- Uses publish–subscribe model, enabling easy scalability for thousands of bins

- Requires very low bandwidth, perfect for LoRa, NB-IoT, and unstable networks

- Supports QoS (Quality of Service) ensuring reliable message delivery

- Faster than HTTP for frequent sensor updates

Example use case:

- ESP32 publishes bin fill percentage to the cloud

- Cloud subscribes to the topic and triggers alerts or analytics

2. HTTP / HTTPS

Where it is used:

- Between Cloud Platform and Web Dashboard / Mobile App

- For fetching reports, visualizations, and system configuration

Why HTTP is suitable:

- Widely supported and easy to implement

- Ideal for request–response communication

- Works well with REST APIs for dashboards and admin panels

- HTTPS ensures secure data transmission

Example use case:

Municipal authority logs in and requests real-time bin status

Dashboard retrieves data via HTTP API

- 3. Route Optimization Strategy

Rule-Based Collection Logic

Initially, a rule-based strategy is applied:

- If bin fill level exceeds 80%, it is marked as high priority

- Alerts are sent to the dashboard

- These bins are added to the collection schedule

Predictive & Data-Driven Optimization (Scalability Enhancement)

To improve efficiency and scalability, the system incorporates data analysis and prediction. Historical fill-level data is analyzed to learn waste accumulation patterns in different areas.

For example:

- A commercial area bin may reach 80% in 2 days

- A residential area bin may take 4–5 days

Using this information, the system predicts future fill levels and schedules garbage collection before overflow occurs.

Route Planning Approach

- Bins predicted to reach critical levels soon are grouped.

- Nearby bins are clustered zone-wise.

- Shortest and most efficient route is suggested for garbage trucks.

This predictive planning minimizes emergency trips and improves operational efficiency.

4. Power Management Plan

Since waste bins are often battery-powered, power efficiency is a key design consideration.

Power Optimization Techniques:

- Periodic sensing instead of continuous sensing

- ESP32 operates in deep sleep mode

- Low-power communication using LoRa

- Optional solar charging for extended deployment

These measures significantly increase battery life and reduce maintenance requirements.

5. Reliability & Fault Handling

Possible Issues:

- Sensor blockage due to plastic or waste

- False readings caused by uneven garbage surface

- Communication failures

Fault Handling Mechanisms:

- Multiple sensor readings are averaged

- Sudden abnormal readings are flagged

- Periodic calibration during maintenance

- Manual verification alerts for inconsistent data

These techniques improve data reliability and system robustness.

6. Scalability & Network Considerations

The system is designed to support 100+ bins across multiple zones.

Network Topology:

- Star topology is used

- Multiple bins connect to a central LoRa gateway

- Scalability Benefits:

- Easy addition of new bins

- Zone-wise expansion

- Centralized monitoring and control

Predictive analytics further enhances scalability by reducing system load and optimizing resource usage.

7. Cost & Feasibility Discussion

| Component | Approx. Cost (INR) |
| --- | --- |
| Ultrasonic Sensor | 150 |
| ESP32 | 400 |
| LoRa Module | 600 |
| Battery & Enclosure | 350 |
| Total | 1500–2000 INR |

Trade-Off Analysis:

- Higher accuracy sensors increase cost

- LoRa has higher initial cost but lower operational expense

- Predictive analytics reduces long-term fuel and manpower costs

- Overall, the system is cost-effective, scalable, and feasible for smart city deployment.